# ESP32-S3 Robot Wiring Guide — Setup 2 (Stepper Motor)

DD38LOSA Regulator | ULN2003 Drivers | 28BYJ-48 Stepper Motors

# 1. Power Regulation & Protection (The Foundation)

Two 18650 batteries in series (7.4V–8.4V) stepped down to a safe 5V rail.

| From | To | Notes |
| --- | --- | --- |
| Battery Pack Positive (+) | SPDT Switch (Middle Pin) |  |
| SPDT Switch (Outer Pin) | DD38LOSA VIN+ (Input Positive) |  |
| Battery Pack Negative (−) | DD38LOSA VIN− or GND (Input Negative) |  |
| 100 µF Capacitor | Across DD38LOSA VOUT+ and VOUT− | Striped leg → VOUT−, longer leg → VOUT+ |

Crucial Note: Capacitors are polarized. Ensure the striped (negative) leg goes to VOUT− and the longer leg to VOUT+. This absorbs voltage spikes from the steppers and prevents the ESP32-S3 from rebooting mid-run.

# 2. Distributing the 5V Rail

Clean, stable 5V shared among all active components.

| Source Pin | Destination Pin | Purpose |
| --- | --- | --- |
| DD38LOSA VOUT+ (5V) | ESP32-S3 5V (or VBUS) | Microcontroller power |
| DD38LOSA VOUT− (GND) | ESP32-S3 GND | Common ground |
| DD38LOSA VOUT+ (5V) | Left ULN2003 Driver + pin | Left driver power |
| DD38LOSA VOUT− (GND) | Left ULN2003 Driver − pin | Left driver ground |
| DD38LOSA VOUT+ (5V) | Right ULN2003 Driver + pin | Right driver power |
| DD38LOSA VOUT− (GND) | Right ULN2003 Driver − pin | Right driver ground |

# 3. ESP32-S3 Logic to ULN2003 Drivers

Each stepper motor requires four GPIO pins to sequence the electromagnets.

## Left Motor Driver

| ESP32-S3 Pin | ULN2003 Pin |
| --- | --- |
| GPIO 1 | IN1 |
| GPIO 2 | IN2 |
| GPIO 3 | IN3 |
| GPIO 4 | IN4 |

## Right Motor Driver

| ESP32-S3 Pin | ULN2003 Pin |
| --- | --- |
| GPIO 9 | IN1 |
| GPIO 10 | IN2 |
| GPIO 11 | IN3 |
| GPIO 12 | IN4 |

Note: GPIO 8 is skipped as it is tied to the built-in LED.

# 4. Final Motor Connections

Plug the white 5-pin plastic connectors into the driver boards.

| Motor | Connection |
| --- | --- |
| Left 28BYJ-48 | White connector → Left ULN2003 board socket |
| Right 28BYJ-48 | White connector → Right ULN2003 board socket |

────────────────────────────────────────────────────────────

Generated Wiring Guide | ESP32-S3 Stepper Motor Controller